| PCB ID: |  |
| --- | --- |

| SINGLE UNIT SIZE (mm): |  | PANEL SIZE (mm): |  | UNITS PER PANEL (PCS): |  |
| --- | --- | --- | --- | --- | --- |

| MANDATORY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Number of layers: | Material type: | PCB thickness: (mm) ±10% | Copper thickness (um) ±10% | Tg |
| ☐ 1 ☐ 2 ☐ 4 ☐ 6 ☐ 8 ☐ …… | ☐ FR4 ☐ CEM1 ☐ Flexible board ☐ Aluminum ☐ …… | ☐ 0.8 ☐ 1.0 ☐ 1.2 ☐ 1.6 ☐ 2.0 ☐ …… | ☐ 17 ☐ 35 ☐ 70 ☐ 105 ☐ see stack-up | ☐ 130 ☐ 150 ☐ 160 ☐ 180 ☐ …… |
| Silk screen printing color Legend print: |  | Solder mask (color): |  | Surface finish: |
| ☐ top ☐ bot ☐ top + bot ☐ WHITE ☐ YELLOW ☐ BLACK ☐ GREEN ☐ …NO silkscreen….. |  | ☐ top ☐ bot ☐ top + bot ☐ GREEN ☐ BLUE ☐ BLACK ☐ …… |  | ☐ Sn/Pb ☐ HASL Lead Free ☐ ENIG ☐ SILVER TREATMENT ☐ …… |

| OPTIONAL |  |  |
| --- | --- | --- |
| Profile: | Impedance controlled tracks: | Blind, Buried Vias: |
| ☐ V-Scoring ☐ Tab route ☐ V-Scoring & Tab route ☐ Punching ☐ …… | ☐ YES ☐ NO | ☐ YES ☐ NO |
| Via Process: | Min trace width/spacing: | Min hole size: |
| ☐ Tenting Vias ☐ Vias Not Covered ☐ Plugged Vias ☐ …… | ☐ 4/4mil (0.1016mm) ☐ 5/5mil (0.127mm) ☐ 6/6mil (0.1524mm) ☐ 8/8mil (0.2032mm) ☐ or higher …… | ☐ 0.2mm ☐ 0.25mm ☐ 0.3mm ☐ 0.35mm ☐ 0.4mm ☐ or higher …… |
| Check: | Marks: |  |
| ☐ OPTIC ☐ MICROSECTION ☐ …… (Mark if required) | ☐ DATE CODE ☐ PRODUCER ☐ VDE Certificat ☐ PB-free marking acc IPC-1066-4-4 ☐ UL94 &796 ☐ Other details: (Mark if required) |  |

| PCB conform of IPC-6012B and IPC-A-600H |  |  |
| --- | --- | --- |
| ☐ Class 1 | ☒ Class 2 (by default) | ☐ Class 3 |

| Approval of working gerber before production: | ☐ YES ☐ NO |
| --- | --- |

Note: RoHS compliant. No nicks, scratches, exposed copper, poor plating or miss drilled holes are accepted. All boards need to be 100% electrically tested.

Special requirements:

| Form filled by (name): |  | Date: |  |
| --- | --- | --- | --- |